물류 시황

엘니뇨 재발에 따른 파나마 운하 통항 리스크 사전 점검

2026.05.29(금) · 삼성SDS

2026년 '슈퍼 엘니뇨' 가능성, 파나마 운하 통항에 어떤 리스크가 있나?

📌 한눈에 보는 핵심요약 NOAA·WMO는 2026년 5~7월 엘니뇨 발생 가능성이 높고, 10월~내년 2월 사이 슈퍼 엘니뇨로 발전할 가능성도 배제할 수 없다고 전망합니다. 엘니뇨에 따른 중남미 강수 감소·가뭄은 갑문식 운하인 파나마 운하의 가툰호수 수위를 떨어뜨려 운영 능력을 직접 제약합니다. '23~'24년에는 최대 허용 흘수가 50→44ft, 일일 통항(전 선종)이 36~38척→22척까지 축소됐습니다. 컨테이너선 통항은 엘니뇨 이전과 유사하게 일평균 약 7척 유지됐으나, 15,000TEU급 기준 최대 약 2,100TEU(-14%)의 적재능력 손실과 저수위 할증료가 화주에게 전가되며 운임 상승 압력으로 작용했습니다. '23년 사례상 영향은 건기 전환기에 본격화되어 올해 연말~내년 초 압박이 심화될 수 있으며, 가툰호수 수위·강수량·통항량·할증료 모니터링과 사전 대응 계획 수립이 필요합니다.

정상 운영 vs 엘니뇨 저수위 심화기 비교 ('23~'24년 기준)

| 구분 | 정상 운영 | 엘니뇨 저수위 심화 |
| --- | --- | --- |
| 최대 허용 흘수 | 50ft | 44ft |
| 일일 통항량 (전 선종) | 36~38척 | 22척 |
| 일일 통항량 (컨테이너선) | 일평균 약 7척 | 일평균 약 7척 |
| 대기 선박 수 | 13.3척 | 17.3척 |
| 대기 시간 | 17.5시간 | 25.8시간 |
| 15,000TEU급 적재능력 | 정상 | 최대 약 -2,100TEU (-14%) |

2026년, '슈퍼 엘니뇨'의 높은 가능성

1) 엘니뇨(El Niño)란?

적도 태평양 감시구역의 해수면온도 3개월 이동평균 편차인 ONI(Oceanic Niño Index)가 +0.5℃ 이상인 상태가 5개월 이상 지속되는 경우를 '엘니뇨'로 정의하며, ONI가 +2.0℃ 이상 장기간 지속될 경우 '슈퍼 엘니뇨'로 구분합니다.

2) 주요 기관 전망

미국 해양대기청(NOAA)의 예측에 따르면, 엘니뇨는 5~7월 사이 발생하고 10월~내년 2월까지는 '슈퍼 엘니뇨'가 발생할 가능성이 높다고 발표했습니다. 세계기상기구(WMO)도 5~7월경 엘니뇨 발생 가능성이 높으며, 중미·남미 북부 등에서 고온·가뭄 경향이 높게 나타날 것으로 전망했습니다.

| 기관 | 전망 |
| --- | --- |
| NOAA | 5~7월 엘니뇨 발생, 10월~내년 2월 '슈퍼 엘니뇨' 발생 가능성 높음 |
| WMO | 5~7월경 엘니뇨 발생 가능성 높음, 중미·남미 북부 고온·가뭄 경향 |

3) 물류 영향

엘니뇨는 해상운송·항만 운영·수자원 관리에 영향을 미치는 주요 기후 리스크로 평가되며, 특히 중남미 지역 강수 감소와 가뭄 발생은 파나마 운하 운영에 직접적인 영향을 미치고 있습니다.

갑문식 운하 구조인 파나마 운하는 선박 1척 통과에 약 2억 리터(올림픽 규격 수영장 80개 규모)의 담수가 필요해, 가툰호수 수위가 파나마 운하 운영 능력을 결정하고 있습니다.

과거 파나마 운하 사례 ('23~'24년)

1) 최대 흘수 및 통항량 제한

'23년 엘니뇨는 13개월 동안 지속되며 가툰호수 수위 하락에 영향을 미쳤습니다. 가툰호수 수위는 엘니뇨 기간 평균 80.5ft, 최저 79.5ft를 기록해, 과거 20년(2000~2019) 평균 85.2ft 대비 매우 낮은 수준이었습니다. 가툰호수 저수위 심화로 최대 허용 흘수는 50→44ft, 일일 통항 선박 수는 36~38척→22척까지 낮춰 운영되었습니다.

2) 운하 혼잡 증가

엘니뇨 초기의 대기 선박 수와 대기 시간은 엘니뇨 이전과 유사했으나, 저수위 누적과 통항 제한이 본격화된 '23년 11월부터 운하 혼잡이 심화되었습니다.

| 구분 | 엘니뇨 이전 ('23.1~4월) | 엘니뇨 초기 ('23.5~10월) | 엘니뇨 심화 ('23.11~'24.3월) |
| --- | --- | --- | --- |
| 대기 선박 수 | 13.3척 | 12.4척 | 17.3척 |
| 대기 시간 | 17.5시간 | 17.1시간 | 25.8시간 |

※ 엘니뇨 초기 대비 심화기 기준 대기 선박 수 +39.2%, 대기 시간 +51.0%

흘수 제한, 통항 슬롯 부족, 비예약 선박 대기 등으로 운영상 병목이 확대되었습니다.

3) 적재량 손실

가툰호수 수위 저하로 일일 통항량은 제한됐으나, 선종에 따라 영향이 다르게 나타났습니다.

| 선종 | 통항량 변화 | 주요 영향 |
| --- | --- | --- |
| 컨테이너선 | 일평균 약 7척 유지 (사전 슬롯 예약) | 흘수 50→44ft 제한 시 15,000TEU급 최대 약 2,100TEU(-14%) 적재능력 감소 |

저수위 영향은 컨테이너선의 통항 횟수 감소보다 운항 유연성 저하, 슬롯 확보 부담 등 비용 증가와 실질 적재능력 감소로 나타났습니다. 일반적으로 적재능력 손실폭은 흘수 제한 구간·선형·항로 등 변동 요인에 따라 달라집니다.

4) 할증료 부과

'23년 하반기부터 선사들은 Canal Surcharge, Low Water Surcharge, Drought Surcharge 등 파나마 운하 저수위에 따른 할증료를 부과하기 시작했습니다.

| 선사 | TEU당 할증료 (부과 당시 기준) |
| --- | --- |
| Maersk | $175 |
| CMA CGM | $150 |
| Hapag-Lloyd | $130 |
| Cosco | $255 |

※ 명칭·금액·적용 시점은 운하 수위·항로·시황 등 변동 요인에 따라 다를 수 있습니다.

파나마 운하의 저수위 영향이 단순 통항 지연을 넘어 추가 선복/항차 운영, 슬롯 확보 비용 등 선사의 운영비용 증가를 할증료 형태로 화주에게 전가하며 운임 상승 압력으로 작용했습니다.

향후 전망

| 핵심 변수 | 시점 | 영향 |
| --- | --- | --- |
| 엘니뇨 지속 가능성 | 올해 여름~내년 초 | 슈퍼 엘니뇨 발전 가능성 배제 못함 |
| 흘수·통항 제한 압박 | 올해 연말~내년 초 | 건기 전환기 본격화로 압박 심화 예상 |
| PCA 통항 정책 | 연내 유지 (일 38척) | 강한 슈퍼 엘니뇨 시 운영 제한 재발 가능 |

- 주요 기상기관은 올해 여름~내년 초까지 엘니뇨가 지속될 가능성이 높으며, 슈퍼 엘니뇨로 발전할 가능성도 배제할 수 없다고 전망합니다.

- '23년 사례를 보면 엘니뇨 영향은 초기보다 강수 부족 누적 및 건기 전환기에 본격화되어 올해 연말~내년 초에 흘수 제한과 일일 통항량 축소로 인한 압박이 심화될 것으로 예상됩니다.

- 현재 통항량은 호르무즈 해협 봉쇄로 인한 미주발 에너지 수요 증가로 늘어난 것이며, 파나마 운하청은 일 38척 통항을 유지하고 연내 제한 계획은 없다고 밝혔으나, 강한 슈퍼 엘니뇨 발생 시 운영 제한 재발 가능성이 존재합니다.

- 가툰호수 수위, 강수량, 일일 통항량, 선사 할증료 변화 등을 지속 모니터링하고, 통항 제한에 대비한 사전 대응 계획 수립이 필요합니다.

자주 묻는 질문

Q. 파나마 운하 통항 제한은 엘니뇨 발생 직후 바로 시작되나요?

A. 아닙니다. 통항 제한은 엘니뇨 발생 직후가 아니라 저수위가 누적된 '23년 11월부터 본격화됐습니다. 엘니뇨 초기('23.5~10월)에는 대기 선박 수·대기 시간이 이전과 유사한 수준이었고, 강수 부족이 누적되는 건기 전환기에 흘수·통항 제한이 강해지는 경향이 있습니다.

Q. 흘수 50→44ft 제한 시 컨테이너선 적재능력은 얼마나 줄어드나요?

A. 흘수가 50ft에서 44ft로 제한되면 15,000TEU급 컨테이너선 기준 최대 약 2,100TEU(약 -14%)의 적재능력이 감소할 수 있습니다. 실제 손실폭은 흘수 제한 구간·선형·항로 등 변동 요인에 따라 달라집니다.

Q. 컨테이너 운임 상승, 통항 척수가 유지되는데도 발생하는 이유는?

A. 통항 척수가 유지돼도 운임 상승 압력이 생기는 이유는 최대 약 -14%의 실질 적재능력 감소와 선사들의 저수위 할증료(Surcharge) 부과 때문입니다. 컨테이너선은 사전 슬롯 예약으로 일평균 약 4척의 통항을 유지했으나, 운항 유연성 저하·슬롯 확보 부담 등 운영비 증가가 Canal·Low Water·Drought Surcharge 형태로 화주에게 전가됐습니다.

Q. 파나마 운하 통항량이 지금 늘었는데도 통항 제한에 대비해야 하나요?

A. 네, 대비가 필요합니다. 현재 통항량 증가는 호르무즈 해협 봉쇄에 따른 미주발 에너지 수요 증가가 주요 원인일 뿐, 엘니뇨 리스크가 해소된 것은 아닙니다. 파나마 운하청은 연내 일 38척 통항 유지 방침이며 제한 계획이 없다고 밝혔으나, 강한 슈퍼 엘니뇨 발생 시 운영 제한이 재발할 가능성이 있습니다.

Q. 파나마 운하 통항 리스크 대응, Cello Square는 어떻게 지원하나요?

A. Cello Square는 삼성SDS가 운영하는 디지털 통합 물류 플랫폼으로, 항로별 총비용(Total Cost) 비교와 실시간 견적을 통해 운하 제한·할증료 변동에 따른 최적 경로 의사결정을 지원합니다. 글로벌 거점 네트워크를 기반으로 가툰호수 수위·통항량 변화에 따른 대체 항로 시나리오를 비교하고 슬롯 확보·납기 관리까지 통합 검토할 수 있습니다.

📎 참고 자료

- NOAA (National Oceanic and Atmospheric Administration) — El Niño / ONI Outlook

- WMO (World Meteorological Organization) — 엘니뇨 발생 및 중미·남미 기후 전망

- 파나마 운하청(PCA) — 흘수·통항 제한 및 통항 정책 발표

- 선사 공지 — Maersk · CMA CGM · Hapag-Lloyd · Cosco 저수위 관련 할증료

- 삼성SDS 물류MI 내부 분석 자료 (2026.05.29)

Cello Square와 함께 운하 통항 리스크에 대응하세요

Cello Square는 삼성SDS가 운영하는 디지털 통합 물류 플랫폼으로, 기후·운하 리스크에 따른 공급망 점검과 총비용 기반 의사결정을 지원합니다. 글로벌 네트워크 기반으로 항로별 물류 견적을 실시간 비교하고, 대체 항로 검토부터 통관 대응까지 통합적으로 해결하세요.

📦 개발팀 전달용 — FAQPage Schema (JSON-LD)

{

"@context": "https://schema.org",

"@type": "FAQPage",

"mainEntity": [

{ "@type": "Question", "name": "파나마 운하 통항 제한은 엘니뇨 발생 직후 바로 시작되나요?",

"acceptedAnswer": { "@type": "Answer", "text": "아닙니다. 통항 제한은 엘니뇨 발생 직후가 아니라 저수위가 누적된 2023년 11월부터 본격화됐습니다. 엘니뇨 초기에는 대기 선박 수·대기 시간이 이전과 유사했고, 강수 부족이 누적되는 건기 전환기에 흘수·통항 제한이 강해지는 경향이 있습니다." } },

{ "@type": "Question", "name": "흘수 50→44ft 제한 시 컨테이너선 적재능력은 얼마나 줄어드나요?",

"acceptedAnswer": { "@type": "Answer", "text": "흘수가 50ft에서 44ft로 제한되면 15,000TEU급 컨테이너선 기준 최대 약 2,100TEU(약 -14%)의 적재능력이 감소할 수 있습니다. 실제 손실폭은 흘수 제한 구간·선형·항로 등 변동 요인에 따라 달라집니다." } },

{ "@type": "Question", "name": "컨테이너 운임 상승, 통항 척수가 유지되는데도 발생하는 이유는?",

"acceptedAnswer": { "@type": "Answer", "text": "통항 척수가 유지돼도 운임 상승 압력이 생기는 이유는 최대 약 -14%의 실질 적재능력 감소와 선사들의 저수위 할증료(Surcharge) 부과 때문입니다. 컨테이너선은 사전 슬롯 예약으로 일평균 약 4척의 통항을 유지했으나, 운영비 증가가 할증료 형태로 화주에게 전가됐습니다." } },

{ "@type": "Question", "name": "파나마 운하 통항량이 지금 늘었는데도 통항 제한에 대비해야 하나요?",

"acceptedAnswer": { "@type": "Answer", "text": "네, 대비가 필요합니다. 현재 통항량 증가는 호르무즈 해협 봉쇄에 따른 미주발 에너지 수요 증가가 주요 원인일 뿐, 엘니뇨 리스크가 해소된 것은 아닙니다. 파나마 운하청은 연내 일 38척 통항 유지 방침이나, 강한 슈퍼 엘니뇨 발생 시 운영 제한이 재발할 가능성이 있습니다." } },

{ "@type": "Question", "name": "파나마 운하 통항 리스크 대응, Cello Square는 어떻게 지원하나요?",

"acceptedAnswer": { "@type": "Answer", "text": "Cello Square는 삼성SDS가 운영하는 디지털 통합 물류 플랫폼으로, 항로별 총비용 비교와 실시간 견적을 통해 운하 제한·할증료 변동에 따른 최적 경로 의사결정을 지원합니다." } }

]

}

}

********************************영문********************************

물류 시황

Pre-check of Panama Canal Transit Risks Due to a Potential El Niño Return

May 29, 2026 | Samsung SDS

What Risks Could a 2026 "Super El Niño" Pose to Panama Canal Transit?

📌 Key Takeaways at a Glance  The National Oceanic and Atmospheric Administration (NOAA) and the World Meteorological Organization (WMO) indicate that the likelihood of an El Niño event developing during May to July 2026 is increasing. They also note that the possibility of it intensifying into a Super El Niño between October 2026 and February 2027 cannot be ruled out.  El Niño-related reductions in rainfall and drought conditions across Central and South America can lower water levels in Gatun Lake, the primary water source for the lock-based Panama Canal. This directly constrains the canal's operating capacity.  During the 2023–2024 El Niño event, the Panama Canal Authority reduced: - Maximum allowable vessel draft from 50 ft to 44 ft - Daily transit capacity (all vessel types) from approximately 36–38 vessels to 22 vessels  While container vessel transits remained relatively stable at around seven vessels per day, similar to pre-El Niño levels, cargo capacity was significantly affected. For a 15,000 TEU-class container ship, the draft restriction resulted in a cargo loss of up to 2,100 TEU (-14%). In addition, low-water surcharges were passed on to shippers, creating upward pressure on freight rates.  Based on the 2023 experience, operational impacts became more pronounced as the region entered the dry season. Similar pressure could intensify during late 2026 and early 2027 if El Niño conditions strengthen.

Comparison: Normal Operations vs. Severe El Niño Low-Water Conditions (Based on 2023–2024 Data)

| Category | Normal Operations | Severe El Niño Low-Water Conditions |
| --- | --- | --- |
| Maximum Allowable Draft | 50ft | 44ft |
| Daily Transit Capacity (All Vessel Types) | 36–38 vessels | 22 vessels |
| Daily Container Ship Transits | Approximately 7 vessels/day | Approximately 7 vessels/day |
| Average Number of Waiting Vessels | 13.3 vessels | 17.3 vessels |
| Average Waiting Time | 17.5 hours | 25.8 hours |
| Cargo Capacity of a 15,000 TEU Container Ship | Full capacity | Up to approximately 2,100 TEU reduction (-14%) |

2026: Elevated Risk of a "Super El Niño"

1) What is El Niño?

El Niño is defined as a climate phenomenon in which the Oceanic Niño Index (ONI), measured as the three-month running average sea surface temperature anomaly in the equatorial Pacific monitoring region, remains at +0.5°C or higher for at least five consecutive months.

When the ONI remains at +2.0°C or higher for an extended period, the event is generally classified as a Super El Niño.

2) Outlook from Major Organizations

According to forecasts from the National Oceanic and Atmospheric Administration (NOAA), an El Niño event is expected to develop between May and July 2026, with a heightened possibility of intensifying into a Super El Niño between October 2026 and February 2027.

The World Meteorological Organization (WMO) also projects a high likelihood of El Niño conditions emerging during May to July, accompanied by an increased risk of above-average temperatures and drought conditions across parts of Central America and northern South America.

| Organization | Outlook |
| --- | --- |
| NOAA | El Niño is expected to develop between May and July, with a high likelihood of intensifying into a Super El Niño between October and February of the following year. |
| WMO | A high probability of El Niño development during May to July, accompanied by an increased tendency toward higher temperatures and drought conditions across Central America and northern South America. |

3) 물류 영향 3) Logistics Impact

El Niño is widely regarded as a major climate risk affecting ocean transportation, port operations, and water resource management. In particular, reduced rainfall and drought conditions across Central and South America can have a direct impact on the operation of the Panama Canal.

As a lock-based canal, the Panama Canal requires approximately 200 million liters of freshwater to move a single vessel through its locks, equivalent to about 80 Olympic-size swimming pools. As a result, the water level of Gatun Lake plays a critical role in determining the canal's operational capacity.

Historical Panama Canal Case (2023–2024)

1) Draft and Transit Restrictions

The 2023 El Niño event lasted for approximately 13 months, contributing to a significant decline in Gatun Lake's water levels.

During the El Niño period, the average water level of Gatun Lake was approximately 80.5 ft, with a low of 79.5 ft, substantially below the 20-year historical average (2000–2019) of 85.2 ft.

As water levels continued to decline, the Panama Canal Authority implemented operational restrictions, reducing:

- Maximum allowable vessel draft: from 50 ft to 44 ft

- Daily transit capacity: from 36–38 vessels to 22 vessels per day

These measures were necessary to conserve water resources while maintaining canal operations under severe drought conditions.

2) 운하 혼잡 증가 2) Increased Canal Congestion

During the early stages of El Niño, the number of waiting vessels and average waiting times remained broadly in line with pre-El Niño levels. However, as low water levels persisted and transit restrictions became more stringent, congestion in the Panama Canal increased significantly from November 2023 onward.

| 구분 Category | Pre-El Niño (Jan–Apr 2023) | Early El Niño (May–Oct 2023) | Severe El Niño Phase (Nov 2023–Mar 2024) |
| --- | --- | --- | --- |
| Average Number of Waiting Vessels | 13.3 vessels | 12.4 vessels | 17.3 vessels |
| Average Waiting Time | 17.5 hours | 17.1 hours | 25.8 hours |

※ Compared with the early El Niño period:  Average number of waiting vessels increased by 39.2%, Average waiting time increased by 51.0%

Operational bottlenecks expanded due to draft restrictions, reduced transit slots, and longer waiting times for non-reserved vessels, resulting in increased canal congestion.

3) 적재량 손실 3) Cargo Capacity Loss

Although daily transit capacity was restricted due to declining water levels in Gatun Lake, the impact varied by vessel type.

| Vessel Type | Change in Transit Volume | Key Impact |
| --- | --- | --- |
| Container Vessels | Maintained at approximately 7 vessels per day (supported by advance slot reservations) | When the maximum allowable draft was reduced from 50 ft to 44 ft, a 15,000 TEU-class container vessel experienced a cargo capacity reduction of up to 2,100 TEU (-14%) |

The impact of low water levels on container shipping was reflected less in the number of vessel transits and more in increased operating costs, reduced scheduling flexibility, greater difficulty securing transit slots, and a decline in effective cargo-carrying capacity.

Actual cargo capacity losses may vary depending on factors such as draft restrictions, vessel design, and trade routes.

4) Surcharge Implementation

Beginning in the second half of 2023, major shipping lines introduced surcharges related to Panama Canal low-water conditions, including Canal Surcharges, Low Water Surcharges, and Drought Surcharges.

| Carrier | Surcharge per TEU (at the time of implementation) |
| --- | --- |
| Maersk | $175 |
| CMA CGM | $150 |
| Hapag-Lloyd | $130 |
| Cosco | $255 |

※ Note: Surcharge names, rates, and implementation dates may vary depending on canal water levels, trade routes, and market conditions.

The impact of low water levels in the Panama Canal extended beyond transit delays. Additional vessel deployment, extra sailings, and higher slot acquisition costs increased carriers' operating expenses, which were subsequently passed on to shippers through surcharges, creating upward pressure on freight rates.

Outlook

| Key Variable | Timing | Potential Impact |
| --- | --- | --- |
| El Niño Persistence | Summer 2026 to Early 2027 | Possibility of developing into a Super El Niño cannot be ruled out |
| Draft and Transit Restrictions | Late 2026 to Early 2027 | Pressure expected to intensify as the region enters the dry season |
| Panama Canal Authority (PCA) Transit Policy | Daily limit of 38 transits currently maintained | Operational restrictions could be reinstated if a strong Super El Niño develops |

  Major meteorological agencies forecast a high likelihood that El Niño conditions will persist from summer 2026 through early 2027, with the potential to strengthen into a Super El Niño.

  Based on the 2023 experience, the most significant impacts are likely to emerge not during the initial phase of El Niño, but after prolonged rainfall deficits accumulate and the region transitions into the dry season. As a result, pressure from draft restrictions and reduced daily transit capacity may intensify during late 2026 and early 2027.

  Current transit volumes have been supported by increased energy shipments to the Americas following disruptions in the Strait of Hormuz. While the Panama Canal Authority has stated that it intends to maintain 38 daily transits and currently has no plans to impose additional restrictions this year, the possibility of renewed operational constraints remains if a severe Super El Niño materializes.

  Continuous monitoring of Gatun Lake water levels, regional precipitation, daily transit volumes, and carrier surcharge developments is recommended, along with the establishment of contingency plans to mitigate potential disruptions caused by future transit restrictions.

Frequently Asked Questions (FAQ)

Q. Do Panama Canal transit restrictions begin immediately after an El Niño event develops?

A. No. Transit restrictions typically do not begin immediately after the onset of El Niño. During the early phase of the 2023 event (May–October 2023), the number of waiting vessels and average waiting times remained similar to pre-El Niño levels. Restrictions became more pronounced from November 2023 onward, after prolonged rainfall deficits led to lower water levels. Historically, draft and transit restrictions tend to intensify as dry-season conditions develop and water shortages accumulate.

Q. How much cargo capacity is lost when the maximum allowable draft is reduced from 50 ft to 44 ft?

A. For a 15,000 TEU-class container vessel, reducing the maximum allowable draft from 50 ft to 44 ft can result in a cargo capacity loss of up to 2,100 TEU (approximately 14%). Actual capacity reductions may vary depending on factors such as vessel design, draft restriction levels, and trade routes.

Q. Why do container freight rates increase even when the number of container vessel transits remains stable?

A. Freight rates can rise even when transit volumes remain stable because of reduced effective cargo capacity and additional carrier costs.

Although container vessels maintained approximately seven transits per day through advance slot reservations, draft restrictions reduced the amount of cargo each vessel could carry by up to 14%. In addition, carriers incurred higher operating costs due to reduced scheduling flexibility, increased competition for transit slots, and other operational constraints. These costs were often passed on to shippers through Canal Surcharges, Low Water Surcharges, and Drought Surcharges, creating upward pressure on freight rates.

Q. Should companies still prepare for potential transit restrictions even though Panama Canal traffic has recently increased?

A. Yes. Preparation remains advisable.

The recent increase in transit volumes has been driven largely by higher energy shipments to the Americas following disruptions in the Strait of Hormuz, rather than by the elimination of El Niño-related risks.

While the Panama Canal Authority has stated that it intends to maintain 38 daily transits and currently has no plans to impose additional restrictions this year, operational constraints could re-emerge if a strong Super El Niño develops.

Q. How does Cello Square support customers in managing Panama Canal transit risks?

A. Cello Square, the digital integrated logistics platform operated by Samsung SDS, helps customers make informed logistics decisions by providing:

- Total logistics cost comparisons across alternative routes

- Real-time freight quotations

- Evaluation of route options under canal restrictions or surcharge changes

- Alternative routing scenarios based on changes in Gatun Lake water levels and canal transit capacity

- Integrated support for transit slot planning and delivery schedule management through Samsung SDS's global logistics network

These capabilities enable shippers to proactively evaluate contingency options and optimize routing decisions when Panama Canal operating conditions change.

📎 References

  NOAA (National Oceanic and Atmospheric Administration) – El Niño and Oceanic Niño Index (ONI) Outlook

  World Meteorological Organization (WMO) – El Niño Development Outlook and Climate Forecasts for Central and Northern South America

  Panama Canal Authority – Announcements on Draft Restrictions, Transit Capacity Adjustments, and Canal Operating Policies

  Carrier Advisories – Low-Water and Drought Surcharges Issued by Maersk, CMA CGM, Hapag-Lloyd, and COSCO Shipping

  Samsung SDS Logistics Market Intelligence (MI) Internal Analysis, May 29, 2026

Respond to Canal Transit Risks with Cello Square

Cello Square, the digital integrated logistics platform operated by Samsung SDS, helps businesses assess supply chain exposure to climate and canal-related disruptions while supporting data-driven, total-cost-based decision making.

Leveraging Samsung SDS's global logistics network, Cello Square enables shippers to compare real-time freight quotations across multiple routes, evaluate alternative routing options, and manage logistics operations more effectively. From contingency planning and route diversification to customs support and execution, the platform provides an integrated approach to navigating supply chain risks associated with weather events, canal restrictions, and evolving market conditions

📦 개발팀 전달용 — FAQPage Schema (JSON-LD)

{

"@context": "https://schema.org",

"@type": "FAQPage",

"mainEntity": [

{ "@type": "Question", "name": "파나마 운하 통항 제한은 엘니뇨 발생 직후 바로 시작되나요?",

"acceptedAnswer": { "@type": "Answer", "text": "아닙니다. 통항 제한은 엘니뇨 발생 직후가 아니라 저수위가 누적된 2023년 11월부터 본격화됐습니다. 엘니뇨 초기에는 대기 선박 수·대기 시간이 이전과 유사했고, 강수 부족이 누적되는 건기 전환기에 흘수·통항 제한이 강해지는 경향이 있습니다." } },

{ "@type": "Question", "name": "흘수 50→44ft 제한 시 컨테이너선 적재능력은 얼마나 줄어드나요?",

"acceptedAnswer": { "@type": "Answer", "text": "흘수가 50ft에서 44ft로 제한되면 15,000TEU급 컨테이너선 기준 최대 약 2,100TEU(약 -14%)의 적재능력이 감소할 수 있습니다. 실제 손실폭은 흘수 제한 구간·선형·항로 등 변동 요인에 따라 달라집니다." } },

{ "@type": "Question", "name": "컨테이너 운임 상승, 통항 척수가 유지되는데도 발생하는 이유는?",

"acceptedAnswer": { "@type": "Answer", "text": "통항 척수가 유지돼도 운임 상승 압력이 생기는 이유는 최대 약 -14%의 실질 적재능력 감소와 선사들의 저수위 할증료(Surcharge) 부과 때문입니다. 컨테이너선은 사전 슬롯 예약으로 일평균 약 4척의 통항을 유지했으나, 운영비 증가가 할증료 형태로 화주에게 전가됐습니다." } },

{ "@type": "Question", "name": "파나마 운하 통항량이 지금 늘었는데도 통항 제한에 대비해야 하나요?",

"acceptedAnswer": { "@type": "Answer", "text": "네, 대비가 필요합니다. 현재 통항량 증가는 호르무즈 해협 봉쇄에 따른 미주발 에너지 수요 증가가 주요 원인일 뿐, 엘니뇨 리스크가 해소된 것은 아닙니다. 파나마 운하청은 연내 일 38척 통항 유지 방침이나, 강한 슈퍼 엘니뇨 발생 시 운영 제한이 재발할 가능성이 있습니다." } },

{ "@type": "Question", "name": "파나마 운하 통항 리스크 대응, Cello Square는 어떻게 지원하나요?",

"acceptedAnswer": { "@type": "Answer", "text": "Cello Square는 삼성SDS가 운영하는 디지털 통합 물류 플랫폼으로, 항로별 총비용 비교와 실시간 견적을 통해 운하 제한·할증료 변동에 따른 최적 경로 의사결정을 지원합니다." } }

]

}

}